FORMAN CHRISTIAN COLLEGE

(A CHARTERED UNIVERSITY)

CSCS 306 A

FA24

Lab 7 Report

Daim Bin Khalid – 251686775

Hafsah Shahbaz - 251684784

21st November 2024

Introduction

This lab aimed to simulate a control system for monitoring the gas volume in a cylinder using an Arduino-based system. The project involved utilizing a potentiometer to simulate volume levels, an LCD to display these values, and LEDs for real-time visual feedback. Task 1 focused on volume display using an LCD, while Task 2 extended this functionality to include a 4-LED setup to indicate volume percentages.

These tasks introduced essential embedded system concepts, including sensor data mapping, LED control, and serial communication for debugging purposes.

Functions

Task 1: LCD-Based Volume Display

The code for Task 1 was designed to:

- Read analog values from a potentiometer.

- Map these values to gas volume (0 to 100 cubic feet).

- Display the mapped volume on a 16x2 LCD in real time.

Task 2: LED-Based Volume Display

Task 2 added 4 LEDs to represent volume percentages:

- 1-25%: Turn ON the 1st LED.

- 26-50%: Turn ON the 1st and 2nd LEDs.

- 51-75%: Turn ON the 1st, 2nd, and 3rd LEDs.

- 76-100%: Turn ON all LEDs.

Each task integrated serial communication to display debugging information, including potentiometer readings and mapped percentages.

Algorithms and Logic

Task 1: LCD Volume Display

Setup Phase:

- LCD Initialization: Pins for the LCD were specified, and the display was initialized using lcd.begin(16, 2).

- Potentiometer Connection: The analog value from the potentiometer was read using analogRead(A0) and mapped to gas volume using the map() function.

Loop Phase:

- Read the potentiometer value.

- Map the value to gas volume (0 to 100 cubic feet).

- Display the mapped value on the LCD.

- Print the raw potentiometer value and the mapped volume to the Serial Monitor.

Task 2: LED Volume Display

Setup Phase:

- LEDs were configured as outputs using pinMode().

Loop Phase:

- Read the potentiometer value and map it to a percentage.

- Control LED states based on the mapped percentage:

  - Turn ON LEDs progressively as volume increases.

  - Turn OFF LEDs when the volume decreases.

- Display real-time readings on the Serial Monitor for debugging.

Code Breakdown

Task 1: LCD-Based Volume Display

#include <LiquidCrystal.h>

// Initialize the LCD (RS, E, D4-D7 pins)

LiquidCrystal lcd(12, 11, 2, 3, 4, 5);

void setup() {

lcd.begin(16, 2); // Initialize the LCD

lcd.setCursor(0, 0);

lcd.print("Gas Volume:"); // Print a test message on the first row

Serial.begin(9600); // Initialize Serial Monitor

}

void loop() {

int potValue = analogRead(A0); // Read potentiometer value

int gasVolume = map(potValue, 0, 1023, 0, 100); // Map to 0-100 cubic feet

lcd.setCursor(0, 1);

lcd.print("Volume: ");

lcd.print(gasVolume);

lcd.print(" c-ft  "); // Add unit

// Print debugging information

Serial.print("Potentiometer Value: ");

Serial.print(potValue);

Serial.print(" | Gas Volume: ");

Serial.println(gasVolume);

delay(500); // Delay for stability

}

Task 2: LED-Based Volume Display

// Define LED pins

const int led1 = 2; // 1st LED (1-25% volume)

const int led2 = 3; // 2nd LED (26-50% volume)

const int led3 = 4; // 3rd LED (51-75% volume)

const int led4 = 5; // 4th LED (76-100% volume)

void setup() {

pinMode(led1, OUTPUT);

pinMode(led2, OUTPUT);

pinMode(led3, OUTPUT);

pinMode(led4, OUTPUT);

Serial.begin(9600); // Initialize Serial Monitor

Serial.println("System Initialized");

}

void loop() {

int potValue = analogRead(A0); // Read potentiometer value

int gasPercentage = map(potValue, 0, 1023, 0, 100); // Map to 0-100%

// Control LEDs based on volume percentage

if (gasPercentage > 0 && gasPercentage <= 25) {

digitalWrite(led1, HIGH);

digitalWrite(led2, LOW);

digitalWrite(led3, LOW);

digitalWrite(led4, LOW);

} else if (gasPercentage <= 50) {

digitalWrite(led1, HIGH);

digitalWrite(led2, HIGH);

digitalWrite(led3, LOW);

digitalWrite(led4, LOW);

} else if (gasPercentage <= 75) {

digitalWrite(led1, HIGH);

digitalWrite(led2, HIGH);

digitalWrite(led3, HIGH);

digitalWrite(led4, LOW);

} else {

digitalWrite(led1, HIGH);

digitalWrite(led2, HIGH);

digitalWrite(led3, HIGH);

digitalWrite(led4, HIGH);

}

// Debugging output

Serial.print("Potentiometer Value: ");

Serial.print(potValue);

Serial.print(" | Gas Percentage: ");

Serial.println(gasPercentage);

delay(500); // Small delay for stability

}

Output

Task 1

Task 2

Serial Monitor Output

Potentiometer Value: 512 | Gas Percentage: 50%

Potentiometer Value: 768 | Gas Percentage: 75%

Potentiometer Value: 1023 | Gas Percentage: 100%

Images